CMPG 325

FINAL TECHNICAL REPORT

Pula Cloud Services Network Implementation

| Document detail | Information |
| --- | --- |
| Project ID | CMPG325-2026-146 |
| Client ID | CLI-146 |
| Client / Organisation | Pula Cloud Services (Kimberley) |
| Assigned Addressing Block | 172.30.96.0/23 |
| Assigned Feature | Wireless security hardening using WPA2-PSK |
| Change Request | CR3 – Isolate Guest Wi-Fi from internal resources |
| Prepared by | Motlalepula Tshabalala |
| Report date | 16 October 2026 |

# 1. Executive Summary

This report documents the design, implementation, and verification of the CMPG325-2026-146 network for Pula Cloud Services in Kimberley. The network is modelled in Cisco Packet Tracer and separates Staff, IT, Servers, Guest Wi-Fi, and Management traffic into distinct VLANs. Inter-VLAN routing is provided by router R1 using 802.1Q subinterfaces, while the switching infrastructure carries the required VLANs between the router, core switch, access switch, and server-room switch.

The assigned Phase 2 feature is wireless security hardening using WPA2-PSK, with AES encryption on both wireless access points. Change Request CR3 requires Guest Wi-Fi to be isolated from Staff, IT, Server, and Management resources. The submitted Phase 2 testing evidence records 15 tests as PASS, including VLAN/trunk checks, gateway and DHCP checks, wireless security, client connectivity, Guest isolation, and preservation of legitimate Staff connectivity.

The report summarises the design and results. The separate testing-evidence document contains the original Packet Tracer screenshots and detailed test evidence; the physical and logical topology files are maintained in the GitHub project repository.

# 2. Project Background, Scope and Objectives

The project models a small organisational network for Pula Cloud Services. The design uses a segmented LAN and separate Staff and Guest wireless networks. Phase 2 focuses on implementing the assigned wireless security feature and verifying that Guest devices cannot reach internal networks.

## 2.1 Objectives

Create separate logical networks for Staff, IT, Servers, Guest Wi-Fi, and Management.

Provide inter-VLAN routing through R1 using 802.1Q subinterfaces.

Carry the required VLANs over the relevant switch trunks.

Provide DHCP addressing to the tested clients and confirm valid addressing and gateway reachability.

Secure both wireless networks using WPA2-PSK with AES encryption.

Apply and verify a Guest isolation access-control policy without breaking tested Staff access to IT and Server resources.

Record implementation and verification evidence in the GitHub portfolio and separate testing report.

## 2.2 Scope and assumptions

The scope documented here is limited to the topology, addressing plan, configuration, and tests evidenced in the project artefacts. This report does not claim that Internet access, production hardware performance, or services beyond those explicitly tested have been validated. Device configuration screenshots and CLI outputs are retained in the separate Phase 2 Testing Documentation.

# 3. Network Design and Topology

The topology follows a centralised switched layout. Router R1 connects to core switch SW1. SW1 provides connectivity to the access-switch segment (SW2), the server-room switch (SW3), and the wireless access points. Wired Staff and IT PCs connect through the access-switch segment. Laptop 1 is the Staff wireless client, and Laptop 2 is the Guest wireless client. Server1 is attached to the server-room segment.

The repository includes editable and exported physical/logical topology artefacts. They should be used alongside this report for the full device layout; the detailed Packet Tracer screenshots are not duplicated here so that the testing evidence remains in its original format.

## 3.1 Device roles

| Device | Role | Purpose in the design |
| --- | --- | --- |
| R1 | Router / Layer 3 gateway | Router-on-a-stick subinterfaces, DHCP evidence, and Guest isolation ACL |
| SW1 | Core switch | Carries required VLANs toward R1, SW2, and SW3; wireless access-port VLAN assignment |
| SW2 | Access switch | Connects wired client devices in the Staff and IT segments |
| SW3 | Server-room switch | Provides switched connectivity for the server-room segment |
| AP1 | Staff wireless access point | SSID PulaCloud-Staff; WPA2-PSK with AES; access port in VLAN 10 |
| AP2 | Guest wireless access point | SSID PulaCloud-Guest; WPA2-PSK with AES; access port in VLAN 40 |
| PC1–PC5 | Wired clients | Staff and IT client addressing and connectivity checks |
| Laptop 1 | Staff wireless client | Staff Wi-Fi association, addressing, and gateway test |
| Laptop 2 | Guest wireless client | Guest Wi-Fi association, addressing, gateway test, and isolation checks |
| Server1 | Server endpoint | Used as a destination for Staff connectivity and Guest isolation testing |

# 4. Logical Network and IP Addressing Plan

The assigned block is 172.30.96.0/23. The following subnets are documented in the Phase 2 testing evidence. The /26 networks provide 62 usable host addresses each, and the /28 Management network provides 14 usable host addresses.

| VLAN | Name | Network / Prefix | Subnet mask | Default gateway | Usable hosts |
| --- | --- | --- | --- | --- | --- |
| 10 | STAFF | 172.30.96.0/26 | 255.255.255.192 | 172.30.96.1 | 62 |
| 20 | IT | 172.30.96.64/26 | 255.255.255.192 | 172.30.96.65 | 62 |
| 30 | SERVERS | 172.30.96.128/26 | 255.255.255.192 | 172.30.96.129 | 62 |
| 40 | GUEST_WIFI | 172.30.96.192/26 | 255.255.255.192 | 172.30.96.193 | 62 |
| 99 | MANAGEMENT | 172.30.97.0/28 | 255.255.255.240 | 172.30.97.1 | 14 |

The separation allows policy to be applied between departments and the Guest network. The unused portion of the assigned /23 block remains available for future expansion, subject to an updated addressing plan.

# 5. Implementation Summary

## 5.1 VLANs and trunking

The logical design uses VLAN 10 for Staff, VLAN 20 for IT, VLAN 30 for Servers, VLAN 40 for Guest Wi-Fi, and VLAN 99 for Management. SW1 trunk verification shows the required VLANs carried toward R1, SW2, and SW3, with the relevant VLANs active and forwarding. The detailed trunk output is Figure 1 in the Phase 2 Testing Documentation.

## 5.2 Inter-VLAN routing

R1 uses router-on-a-stick routing. 802.1Q subinterfaces provide the default gateways for VLANs 10, 20, 30, 40, and 99. The recorded verification shows the configured subinterfaces in the up/up state. Refer to Figure 2 in the testing evidence for the interface and gateway output.

## 5.3 DHCP and client addressing

The recorded DHCP binding output confirms valid dynamic addressing for the tested Staff and IT PCs. The Staff wireless client received 172.30.96.13/26 and reached gateway 172.30.96.1 with 0% packet loss. The Guest wireless client received 172.30.96.201/26 and reached gateway 172.30.96.193. The corresponding screenshots are Figures 3, 7, and 8 in the testing evidence.

## 5.4 Wireless configuration

| Device | SSID | Network assignment | Authentication | Encryption |
| --- | --- | --- | --- | --- |
| AP1 | PulaCloud-Staff | VLAN 10 / Staff | WPA2-PSK | AES |
| AP2 | PulaCloud-Guest | VLAN 40 / Guest Wi-Fi | WPA2-PSK | AES |

The testing evidence identifies SW1 Fa0/22 as the AP1 Staff access port in VLAN 10 and SW1 Fa0/23 as the AP2 Guest access port in VLAN 40. Both access points use WPA2-PSK authentication with AES encryption. Pre-shared keys are intentionally omitted from this report.

## 5.5 Guest isolation policy

The GUEST_ISOLATION extended access-control list on R1 is applied inbound to the Guest VLAN subinterface. The ACL first permits DHCP client-to-server traffic so that Guest wireless clients can obtain valid IP addressing. It then contains explicit deny statements preventing Guest traffic from reaching the Staff, IT, Server, and Management networks, followed by a permit rule for other Guest traffic. The policy was verified by confirming that the Guest wireless client could obtain a valid Guest address and reach its own gateway while access to protected internal destinations remained blocked.

# 6. Verification Method and Test Results

Verification used Packet Tracer device configuration windows, switch/router CLI output, DHCP binding information, client IP configuration, ping tests, and ACL counters. The detailed screenshots are contained in the separate Phase 2 Testing Documentation, Figures 1–12.

| ID | Test | Expected / verified result | Status |
| --- | --- | --- | --- |
| T01 | Core trunk operation | SW1 trunk ports; required VLANs active/forwarding | PASS |
| T02 | Router-on-a-stick gateways | R1 subinterfaces for VLANs 10, 20, 30, 40, and 99 up/up | PASS |
| T03 | DHCP allocation | Valid Staff and IT client leases recorded | PASS |
| T04 | AP VLAN assignment | AP1 access port in VLAN 10; AP2 access port in VLAN 40 | PASS |
| T05 | Staff WPA2-PSK security | AP1 uses WPA2-PSK with AES | PASS |
| T06 | Guest WPA2-PSK security | AP2 uses WPA2-PSK with AES | PASS |
| T07 | Staff wireless connectivity | Laptop 1 has valid Staff address and reaches 172.30.96.1 | PASS |
| T08 | Guest wireless connectivity | Laptop 2 has valid Guest address and reaches 172.30.96.193 | PASS |
| T09 | Guest-to-Staff isolation | Guest access to Staff network blocked | PASS |
| T10 | Guest-to-IT isolation | Guest access to IT network blocked | PASS |
| T11 | Guest-to-Server isolation | Guest access to Server network blocked | PASS |
| T12 | Guest-to-Management isolation | Guest access to 172.30.97.1 blocked | PASS |
| T13 | Staff-to-IT connectivity | Staff client can reach IT network | PASS |
| T14 | Staff-to-Server connectivity | Staff client can reach Server1 | PASS |
| T15 | Guest ACL operation | GUEST_ISOLATION ACL counters record matches | PASS |

Outcome: all 15 tests listed in the Phase 2 testing summary are recorded as PASS. This supports the conclusion that the documented VLAN segmentation, gateway routing, tested DHCP allocation, wireless security settings, Guest isolation, and selected Staff connectivity checks operate as intended in the Packet Tracer model.

# 7. Security Considerations and Recommendations

The implemented controls documented in the project are:

Logical separation of Staff, IT, Servers, Guest Wi-Fi, and Management traffic using VLANs.

WPA2-PSK authentication with AES encryption on both wireless access points.

ACL-based blocking of Guest access to Staff, IT, Server, and Management networks.

Verification that the Guest policy does not block the tested Staff-to-IT and Staff-to-Server traffic.

Recommended improvements beyond the tests documented for this submission:

Use unique, strong wireless pre-shared keys and rotate them if they have been exposed in screenshots or public repository files.

Keep credentials and other secrets out of public GitHub repositories; redact only the sensitive field while preserving the original screenshot context.

Restrict administrative access to network devices and the Management VLAN to authorised IT devices.

Maintain backups of the Packet Tracer file and device configurations, and document changes with clear commit messages.

If required by the client, add separate tests for Internet reachability, DNS, server application availability, and resilience/failure scenarios before claiming those services are verified.

# 8. Submission Artefacts and Repository Structure

The following artefacts should be submitted together and kept consistent with the final working Packet Tracer project. File names below follow the repository layout visible during project preparation; verify the exact final names on GitHub before submission.

| Artefact | Expected location / filename | Purpose |
| --- | --- | --- |
| Packet Tracer project | Phase-2/Packet-Tracer/CMPG325-2026-146_Phase2_Final.pkt | Working network model |
| Phase 2 testing evidence | Phase-2/Testing-Evidence/Phase2_Testing evidence.docx | Original screenshots and detailed PASS results |
| Phase 2 README | Phase-2/README.md | Implementation summary and requirements |
| Physical topology | Pula_Cloud_Physical_Topology.drawio and exported PNG | Physical network layout |
| Logical topology | Pula_Cloud_logical_Topology.drawio and exported image(s) | VLAN and logical network design |
| Main README | README.md | Project overview and status |
| Final technical report | CMPG325_Final_Technical_Report.docx | This report |

GitHub repository: https://github.com/Motlalepula-Tshabalala/CMPG325-2026-Pula-Cloud-Network

# 9. Conclusion

The project artefacts document a segmented network for Pula Cloud Services using VLANs, router-on-a-stick routing, DHCP, secured Staff and Guest wireless networks, and an ACL to isolate Guest clients from internal resources. The Phase 2 testing summary records all 15 defined tests as PASS, including evidence that the Guest client can reach its own gateway but is blocked from Staff, IT, Server, and Management resources, while tested Staff access to IT and Server resources remains available.

The Packet Tracer file has been opened and checked by the student, with the topology and connections reported as working. Final readiness therefore depends on reviewing the report, confirming that the latest evidence and project file are present in GitHub, checking that no Wi-Fi credentials are exposed in the public repository, and completing the required INSET demonstration video.

# References and project evidence

CMPG325_Phase2_Testing_Documentation_FINAL.docx, 15 pages, including test summary and Figures 1–12.

Cisco Packet Tracer project file: CMPG325-2026-146_Phase2_Final.pkt.

Project physical and logical topology files in the CMPG325-2026-Pula-Cloud-Network GitHub repository.

Final-submission instructions supplied for CMPG325: Packet Tracer project, GitHub portfolio of evidence, technical report and INSET video demonstration.