CMPG 325 – COMPUTER NETWORKS

PHASE 2 – CLIENT IMPLEMENTATION REVIEW

Testing Documentation and Evidence

| Project ID | CMPG325-2026-146 |
| --- | --- |
| Client ID | CLI-146 |
| Organisation | Pula Cloud Services (Kimberley) |
| Assigned Addressing Block | 172.30.96.0/23 |
| Assigned Feature | Wireless Security – WPA2-PSK hardening |
| Change Request | CR3 – Guest Wi-Fi isolated from internal resources |
| Date | 1 October 2026 |

# 1. Test Scope

The Phase 2 implementation was tested to confirm VLAN operation, inter-VLAN routing, DHCP addressing, WPA2-PSK wireless security, wireless client connectivity, and enforcement of the Guest Wi-Fi isolation requirement. Packet Tracer captures show device identity, configuration context, and actual test output

## 1.1 Implemented Logical Networks

| VLAN | Name | Network | Gateway |
| --- | --- | --- | --- |
| 10 | STAFF | 172.30.96.0/26 | 172.30.96.1 |
| 20 | IT | 172.30.96.64/26 | 172.30.96.65 |
| 30 | SERVERS | 172.30.96.128/26 | 172.30.96.129 |
| 40 | GUEST_WIFI | 172.30.96.192/26 | 172.30.96.193 |
| 99 | MANAGEMENT | 172.30.97.0/28 | 172.30.97.1 |

## 1.2 Test Summary

| ID | Test | Source / Area | Verified Result | Status | Evidence |
| --- | --- | --- | --- | --- | --- |
| T01 | Core trunk operation | SW1 | Required trunks and VLANs active/forwarding | PASS | Fig. 1 |
| T02 | Router-on-a-stick gateways | R1 | All VLAN subinterfaces up/up | PASS | Fig. 2 |
| T03 | DHCP allocation | R1 / PCs | Valid Staff and IT leases issued | PASS | Fig. 3 |
| T04 | AP VLAN assignment | SW1 | AP1 in VLAN 10; AP2 in VLAN 40 | PASS | Fig. 4 |
| T05 | Staff WPA2-PSK security | AP1 | PulaCloud-Staff uses WPA2-PSK with AES | PASS | Fig. 5 |
| T06 | Guest WPA2-PSK security | AP2 | PulaCloud-Guest uses WPA2-PSK with AES | PASS | Fig. 6 |
| T07 | Staff wireless connectivity | Laptop1 | Valid Staff address; gateway reachable | PASS | Fig. 7 |
| T08 | Guest wireless connectivity | Laptop2 | Valid Guest address; gateway reachable | PASS | Fig. 8 |
| T09 | Guest to Staff isolation | Laptop2 | Blocked | PASS | Fig. 9 |
| T10 | Guest to IT isolation | Laptop2 | Blocked | PASS | Fig. 9 |
| T11 | Guest to Server isolation | Laptop2 | Blocked | PASS | Fig. 9 |
| T12 | Guest to Management isolation | Laptop2 | Blocked | PASS | Fig. 10 |
| T13 | Staff to IT connectivity | Laptop1 | Reachable | PASS | Fig. 11 |
| T14 | Staff to Server connectivity | Laptop1 | Reachable | PASS | Fig. 11 |
| T15 | Guest ACL operation | R1 | ACL counters record matches | PASS | Fig. 12 |

# 2. Implementation and Test Evidence

## 2.1 Core VLAN Trunk Verification

SW1 carries the required VLANs toward R1, SW2, and SW3. The device title remains visible, confirming that the evidence comes from the Core Switch.

Figure 1: SW1 trunk interfaces, allowed VLANs, and forwarding VLANs.

## 2.2 Router-on-a-Stick Interface Verification

R1 provides the default gateways for VLANs 10, 20, 30, 40, and 99 through 802.1Q subinterfaces. All configured subinterfaces are shown in the up/up state.

Figure 2: R1 subinterfaces and gateway IP addresses.

## 2.3 DHCP Address Allocation

The R1 DHCP binding table confirms that Staff and IT clients received valid dynamic addresses from the correct address pools.

Figure 3: R1 DHCP bindings for PC1–PC5.

## 2.4 Wireless Access-Port VLAN Assignment

SW1 separates the wireless networks at the access-port level: Fa0/22 belongs to VLAN 10 (Staff) and Fa0/23 belongs to VLAN 40 (Guest Wi-Fi).

Figure 4: SW1 VLAN membership for the AP1 and AP2 access ports.

## 2.5 Staff WPA2-PSK Security

AP1 is configured with SSID PulaCloud-Staff, WPA2-PSK authentication, and AES encryption.

Figure 5: AP1 – Staff Wireless AP configured for WPA2-PSK with AES.

## 2.6 Guest WPA2-PSK Security

AP2 is configured with SSID PulaCloud-Guest, WPA2-PSK authentication, and AES encryption.

Figure 6: AP2 – Guest Wireless AP configured for WPA2-PSK with AES.

## 2.7 Staff Wireless Connectivity

Laptop1 received address 172.30.96.13/26 and successfully reached the Staff gateway 172.30.96.1 with 0% packet loss.

Figure 7: Laptop 1 – Staff wireless client addressing and successful gateway test.

## 2.8 Guest Wireless Connectivity

Laptop2 received address 172.30.96.201/26 and successfully reached the Guest gateway 172.30.96.193.

Figure 8: Laptop 2 – Guest wireless client addressing and successful Guest gateway test.

## 2.9 Guest Isolation from Staff, IT and Servers

After GUEST_ISOLATION was applied, the Guest client could not reach Staff, IT, or Server destinations.

Figure 9: Guest client blocked from Staff, IT, and Server resources.

## 2.10 Guest Isolation from Management

The Guest client was also prevented from reaching the VLAN 99 management gateway 172.30.97.1.

Figure 10: Guest client blocked from the Management network.

## 2.11 Internal Connectivity Preserved

The security policy applies to Guest traffic only. The Staff wireless client can still reach the IT network and Server1.

Figure 11: Staff wireless client successfully reaches IT and Server resources.

## 2.12 ACL Verification

The R1 GUEST_ISOLATION ACL contains explicit deny statements for Staff, IT, Server, and Management networks, followed by a permit-to-any rule. Match counters confirm that Guest test traffic was processed by the ACL.

Figure 12: R1 GUEST_ISOLATION ACL configuration and packet match counters.

# 3. Test Outcome

The Phase 2 implementation passed all recorded tests. Staff and IT clients receive valid DHCP addressing; VLAN routing operates through R1; AP1 and AP2 use WPA2-PSK with AES; Staff and Guest wireless clients connect to their correct networks; and the Guest VLAN is isolated from Staff, IT, Server, and Management resources while legitimate internal connectivity remains available.